AUTONOMOUS SAFETY RC CAR

PIC16F877A  ·  Raspberry Pi  ·  Firebase  ·  Android

https://v0-rc-car-dashboard.vercel.app/

https://lab1-f7c43-default-rtdb.firebaseio.com/

Technical Project Report

Figure 1 — Assembled RC Car prototype

Ahmed Salah 202200212

Ahmed Mohamed 202200294

Abdelhady Mohamed 202201172

Youssef Mohamed 202200286

Ahmed Yasser 202200477

# Table of Contents

Table of Contents	2

1. Introduction	3

1.1 Objectives	3

2. System Architecture	3

2.1 Data Flow	3

3. Hardware Design	4

3.1 Component List	4

3.2 Pin Connections	4

Motor Driver (L298N)	4

HC-SR04 Ultrasonic Sensors	5

LCD (I2C PCF8574 Backpack)	5

Bluetooth / UART (RC7 = TX, RC6 = RX on PIC)	5

Safety Sensors	5

3.3 PCB Design	5

4. PIC Firmware (main.c)	7

4.1 Project Architecture (Layered Driver Model)	7

4.2 Safety State Machine	7

4.3 Autonomous Obstacle Avoidance	7

4.4 UART Command Interface	8

5. Raspberry Pi Software	9

5.1 final.py — Combined UART/Firebase Bridge	9

5.2 eye_to_pic.py — Eye Monitor Relay	9

5.3 lane_detection_firebase.py — Vision Pipeline	9

6. Firebase Realtime Database	9

7. Next.js Web Dashboard	11

8. Physical Build	12

9. Power Supply	13

10. Testing & Results	14

11. Future Work	14

12. Conclusion	14

# 1. Introduction

This report documents the design, implementation, and testing of an autonomous safety RC car. The system demonstrates how embedded microcontroller firmware, computer vision, cloud connectivity, and a web dashboard can be combined into a single cohesive platform.

The primary objective was to build a self-driving RC car that enforces real-world automotive safety rules — door status, seatbelt status, and driver attentiveness — before allowing the vehicle to move. A PIC16F877A microcontroller serves as the real-time control core, while a Raspberry Pi handles vision and cloud communication.

## 1.1 Objectives

- Implement fully autonomous obstacle-avoidance using three HC-SR04 ultrasonic sensors

- Enforce a layered safety state machine (door → seatbelt → eyes-open) with motor kill-switch

- Detect lane markings via Raspberry Pi camera and publish offset data to Firebase

- Monitor driver eye state on an Android device using ML and relay commands to PIC over UART

- Stream all telemetry to a Firebase Realtime Database and display it on a Next.js dashboard

- Design and manufacture a custom PCB to consolidate all connections

# 2. System Architecture

The system is composed of five interacting layers:

- PIC16F877A firmware — real-time safety state machine and motor control

- Raspberry Pi — UART bridge, lane-detection vision pipeline, Firebase I/O

- Android app — driver eye monitoring via on-device ML, Firebase publisher

- Firebase Realtime Database — shared cloud message bus

- Next.js dashboard — browser-based monitoring and manual control UI

## 2.1 Data Flow

The diagram below summarises how data moves between components:

┌─────────────┐   UART 9600    ┌──────────────────────────────┐

│  PIC16F877A │◄──────────────►│       Raspberry Pi           │

│  (main.c)   │  telemetry /   │  final.py  │  lane_det.py    │

└──────┬──────┘  eye cmds      └─────┬──────┴────────┬────────┘

│                             │               │

Motors,                      Firebase       Firebase

LCD, Sensors              /car_telemetry  /lane_status

│               │

┌──────▼───────────────▼──────┐

│   Firebase Realtime Database │

└──────┬───────────────┬───────┘

│               │

Next.js Dashboard   Android App

(monitoring/ctrl)  /eye_monitor

# 3. Hardware Design

## 3.1 Component List

| Component | Quantity | Role |
| --- | --- | --- |
| PIC16F877A | 1 | Main microcontroller (firmware) |
| Raspberry Pi (Zero 2W / 3B+) | 1 | Vision + Firebase bridge |
| HC-SR04 Ultrasonic Sensor | 3 | Front / Left / Right obstacle detection |
| L298N H-Bridge Module | 1 | DC motor driver |
| DC Gear Motors | 4 | Wheel drive (4WD chassis) |
| HC-05 Bluetooth Module | 1 | Optional wireless serial (legacy) |
| LCD 16×2 with I2C PCF8574 | 1 | On-car status display |
| Magnetic Door Switch | 1 | Door-open safety interlock (RA0) |
| Seatbelt Push-Button | 1 | Seatbelt safety interlock (RA1) |
| 20 MHz Crystal + 22 pF caps | 1 set | PIC oscillator |
| Anker USB Power Bank | 1 | Raspberry Pi power supply |
| Custom PCB (EasyEDA) | 1 | PCB consolidating all connections |

## 3.2 Pin Connections

The following table reflects the actual connections used on the physical car:

### Motor Driver (L298N)

| PIC Pin | L298N Pin | Function |
| --- | --- | --- |
| RD4 | IN1 | Motor A — Left wheel forward |
| RD5 | IN2 | Motor A — Left wheel reverse |
| RD6 | IN3 | Motor B — Right wheel forward |
| RD7 | IN4 | Motor B — Right wheel reverse |

### HC-SR04 Ultrasonic Sensors

| Sensor | TRIG Pin | ECHO Pin |
| --- | --- | --- |
| Front | RB1 | RB2 |
| Left | RD0 | RD1 |
| Right | RB3 | RB4 |

### LCD (I2C PCF8574 Backpack)

| Signal | PIC Pin | Pull-up |
| --- | --- | --- |
| SDA | RC4 (pin 23) | 4.7 kΩ to +5V |
| SCL | RC3 (pin 18) | 4.7 kΩ to +5V |

### Bluetooth / UART (RC7 = TX, RC6 = RX on PIC)

| Signal | PIC Pin | Notes |
| --- | --- | --- |
| TX | RC7 | UART RX from HC-05 / Raspberry Pi |
| RX | RC6 | 1kΩ+2kΩ divider to protect 3.3V targets |

### Safety Sensors

| Sensor | PIC Pin | Logic (with 10 kΩ pull-up to VCC) |
| --- | --- | --- |
| Magnetic door switch | RA0 | 0 = door closed (safe), 1 = door open (danger) |
| Seatbelt switch | RA1 | 1 = belt fastened (safe), 0 = unbuckled (danger) |

## 3.3 PCB Design

A custom two-layer PCB was designed in EasyEDA to consolidate all signal routing, decouple power rails, and reduce breadboard wiring errors. The board hosts the PIC16F877A in a DIP-40 socket, the crystal oscillator circuit, UART level-shifting resistors, and female headers for all peripheral connectors.

Figure 2 — Custom PCB layout (EasyEDA)

Figure 3 — Full circuit schematic

# 4. PIC Firmware (main.c)

## 4.1 Project Architecture (Layered Driver Model)

The firmware follows a strict MCAL → HAL → APP layered architecture, commonly used in professional automotive embedded software:

| Layer | Location | Responsibility |
| --- | --- | --- |
| APP | APP/main.c | System integration, state machine, main loop |
| HAL | HAL/Motor, HAL/HC-SR04, HAL/LCD, HAL/Bluetooth, HAL/Switch | Peripheral abstraction |
| MCAL | MCAL/UART, MCAL/GPIO, MCAL/I2C, MCAL/PWM, MCAL/TMR1, MCAL/EXT_INT | Register-level drivers |
| SERVICES | SERVICES/STD_TYPES.h, BIT_MATH.h, config.h | Shared types and macros |

## 4.2 Safety State Machine

The firmware enforces four mutually-exclusive system states, evaluated in priority order on every loop iteration. Any unsafe condition immediately calls Motor_Stop() and shows an alert on the LCD.

| State | Trigger Condition | LCD Line 0 | Motor Action |
| --- | --- | --- | --- |
| STATE_DOOR_OPEN | RA0 == 1 (door open) | "!! DOOR OPEN !!" | Motor_Stop() |
| STATE_NO_BELT | RA1 == 0 (belt unfastened) | "!! NO SEATBELT !" | Motor_Stop() |
| STATE_EYES_CLOSED | eyes_open == 0 (UART 'C') | "!! DROWSY DRIVER" | Motor_Stop() |
| STATE_SAFE | All checks pass | "F:xx L:xx R:xx" | Autonomous driving |

## 4.3 Autonomous Obstacle Avoidance

When in STATE_SAFE and MODE_AUTO, the firmware samples all three ultrasonic sensors and applies the following decision tree (OBSTACLE_DIST = 30 cm):

- Front > 30 cm: Motor_Forward()

- Front ≤ 30 cm, Left > Right and Left > 30 cm: Motor_Turn_Left() for 400 ms

- Front ≤ 30 cm, Right > Left and Right > 30 cm: Motor_Turn_Right() for 400 ms

- All directions blocked: Motor_Reverse() 600 ms → Motor_Turn_Right() 600 ms (escape spin)

Three fixed sensors (rather than a single servo-mounted sensor) were chosen to eliminate Timer1 conflicts between servo PWM ISR and ultrasonic echo timing, and to remove inrush current spikes from servo movement.

## 4.4 UART Command Interface

| Byte | Source | Effect |
| --- | --- | --- |
| 'E' | Raspberry Pi (eye relay) | eyes_open = 1 → unblock STATE_SAFE |
| 'C' | Raspberry Pi (eye relay) | eyes_open = 0 → STATE_EYES_CLOSED, Motor_Stop() |
| 'A' | Dashboard / Pi | Switch to MODE_AUTO |
| 'M' | Dashboard / Pi | Switch to MODE_MANUAL, Motor_Stop() |
| 'F' | Manual mode | Motor_Forward() |
| 'B' | Manual mode | Motor_Reverse() |
| 'L' | Manual mode | Motor_Turn_Left() |
| 'R' | Manual mode | Motor_Turn_Right() |
| 'S' | Manual mode | Motor_Stop() |

# 5. Raspberry Pi Software

## 5.1 final.py — Combined UART/Firebase Bridge

final.py is the primary production script. It performs two tasks in a single 10 Hz loop:

- Reads PIC telemetry lines (F:xxx R:xxx L:xxx D:x B:x) from UART and pushes structured JSON to Firebase /car_telemetry at up to 5 Hz

- Polls Firebase /eye_monitor every 500 ms and sends 'E' or 'C' to the PIC when the eye state changes

The script includes fail-safe shutdown (SIGINT/SIGTERM sends 'C') and dry-run mode for testing without hardware.

# Run in production:

sudo python3 final.py

# Test without UART hardware:

python3 final.py --dry-run -v

## 5.2 eye_to_pic.py — Eye Monitor Relay

eye_to_pic.py is a standalone script that polls the Firebase /eye_monitor path (published by the Android app) and relays 'E'/'C' bytes to the PIC. It handles stale data detection (STALE_MS = 2000), exponential backoff on network failure, and a 5-second heartbeat re-send.

## 5.3 lane_detection_firebase.py — Vision Pipeline

This script captures frames from the PiCamera2 at 640×480, applies the following pipeline, and publishes results to Firebase /lane_status:

- Greyscale conversion and binary threshold (pixels < 90 = black tape)

- Morphological opening with 15×15 kernel to eliminate grout-line noise

- Row-scan from bottom to ROI top, finding the widest gap between tape strips

- Outlier rejection using median absolute deviation

- Weighted quadratic fit (x = f(y)) with bottom rows weighted more

- EMA temporal smoothing (α = 0.20) for stable centre-line tracking

- Pixel offset from frame centre pushed to Firebase; displayed on OpenCV overlay

# 6. Firebase Realtime Database

Firebase serves as the shared message bus between all system components. Three top-level nodes are used:

| Path | Writer | Reader(s) | Update Rate |
| --- | --- | --- | --- |
| /car_telemetry | final.py (Pi) | Dashboard, Android app | ≤5 Hz |
| /eye_monitor | Android app | final.py, eye_to_pic.py | ≤10 Hz |
| /lane_status | lane_detection_firebase.py | Dashboard | ≤5 Hz |

The telemetry node stores: front/left/right distance (cm), door status (OPEN/CLOSED), belt status (FASTENED/UNBUCKLED), and a server-side timestamp. The eye_monitor node stores: state (OPEN/CLOSED/UNKNOWN), probability scores, FPS, and timestamp.

# 7. Next.js Web Dashboard

The web dashboard is built with Next.js 14 and Shadcn/UI components. It subscribes to all three Firebase paths and renders live telemetry including:

- Ultrasonic distance gauges (Front / Left / Right) with obstacle warning indicators

- Safety status panel (door, seatbelt, eye state) with colour-coded alerts

- Drive mode selector (AUTO / MANUAL) sending UART commands via Firebase

- Lane offset visualiser showing centre-line deviation in real time

- Connection health indicator (Firebase latency and last-update timestamp)

cd rc-car-dashboard

pnpm install

pnpm dev          # http://localhost:3000

# 8. Physical Build

The RC car chassis is a standard 4WD wooden/acrylic platform. The custom PCB is mounted centrally on the top layer, with the Raspberry Pi and Anker power bank occupying the upper deck. Ultrasonic sensors are mounted at front, left, and right via 3D-printed brackets. The LCD is mounted upright on the rear of the upper deck for visibility.

Figure 4 — Side view showing PCB, Raspberry Pi, and wiring

Figure 5 — Top view showing LCD display and Anker power bank

# 9. Power Supply

| Channel | Voltage | Powers |
| --- | --- | --- |
| Ch. 1 | +5V | PIC, HC-SR04 ×3, LCD I2C, HC-05, Raspberry Pi |
| Ch. 2 | +5–12V | L298N motor supply (depends on motor rating) |
| Ch. 3 | +5V | Servo signal (optional — separate from PIC rail) |

All three channels share a common GND bus. Mandatory decoupling: 100 nF ceramic at PIC VDD pins, each HC-SR04, and the PCF8574; 100 µF electrolytic at L298N motor input.

# 10. Testing & Results

| Test | Method | Result |
| --- | --- | --- |
| Obstacle avoidance | Placed objects at < 30 cm on each sensor axis | Car stopped and navigated around all obstacles |
| Door safety interlock | Opened magnetic door switch mid-run | Motors stopped within one loop cycle (~50 ms) |
| Seatbelt interlock | Released seatbelt button during autonomous run | Motors stopped; LCD showed NO SEATBELT alert |
| Eye-close detection | Covered Android camera mid-run | UART 'C' received; STATE_EYES_CLOSED triggered; car stopped |
| Lane detection | Ran car along black duct-tape lane | Offset pushed to Firebase; centre-line visible on OpenCV overlay |
| UART telemetry | Monitored Firebase /car_telemetry during run | 5 Hz updates received; all fields populated correctly |
| Dashboard live update | Opened dashboard during autonomous run | All gauges and status indicators updated in real time |

# 11. Future Work

- PID-controlled lane following using the lane-offset signal from the Pi

- Servo-mounted single-sensor scanning mode (ISR-safe path already implemented in firmware)

- Speed control via PWM on L298N ENA/ENB pins using PIC CCP modules

- OTA firmware updates over Firebase → Pi → UART bootloader

- Battery voltage monitoring with ADC and low-battery alert on LCD

- Dashboard authentication and multi-car fleet management

# 12. Conclusion

This project successfully integrates a PIC16F877A real-time controller, Raspberry Pi vision and cloud bridge, Android eye-monitoring, and a web dashboard into a unified autonomous safety car platform. The layered firmware architecture keeps each driver independent and testable. The use of Firebase as a shared message bus decouples all components so any layer can be updated or replaced without affecting the others.

The most significant technical achievement is the four-layer safety state machine that enforces door, seatbelt, and driver-attentiveness checks in priority order, guaranteeing that motors cannot run unless all safety conditions are met simultaneously — a pattern directly applicable to real automotive systems.

— End of Report —